# Jetson Orin Setup Manual

Table of Contents

Jetson Orin Setup Manual	1

1.	Upgrade Jetpack using SDK Manager	1

1.1.	Prepare a host	1

1.2.	Download SDK Manager	2

1.3.	Connect Jetson Orin to host and put the board into Recovery Mode	2

1.3.1.	Connect Jetson Orin to host	2

1.3.2.	Put Jetson Orin into Recovery Mode	3

1.3.3.	(For WSL2 only) Connect USB devices to WSL2	3

1.3.4.	Confirm if host is successfully connected to Jetson Orin in Recovery Mode	3

1.4.	Upgrade using SDK Manager	3

1.4.1.	Flash OS	4

1.4.2.	Login into Jetson Orin and Initialize the OS. Connect Jetson to a Wi-Fi / Ethernet.	5

1.4.3.	Install Jetson SDK Components through Ethernet using SDK Manager	5

1.5.	Check if Jetpack OS is successfully installed	6

1.6.	Check if Jetpack SDK Components are successfully installed	6

2.	Configurate Additional SSD	7

2.1.	Formula the SSD	7

2.2.	Change Mount Point	8

2.3.	Start the SSD	9

2.4.	Verify SSD Performance	9

2.5.	Access SSD	11

3.	Install RT-Kernel	11

3.1.	Verify if RT-Kernel is installed	11

All information is referenced to Jetpack6.2 and Jetson Orin with SDK Manager v2.2.0.

Author: Ruby Lam

Last Edit Date: 2/4/2025

## Upgrade Jetpack using SDK Manager

### Prepare a host

SDK Manager need to be installed on a separate x86-64(Amd64) host and flash to the Jetson device with a USB Type-C cable.

For Jetpack6.2, Ubuntu 20.04 / 22.04 / 24.04 is supported.

See https://developer.nvidia.com/sdk-manager#host_os_comp_matrix

### Download SDK Manager

- Download Ubuntu .deb from here.

- Install the SDK Manager with command

“sudo dpkg -i sdkmanager_2.2.0-12028_amd64.deb”

### Connect Jetson Orin to host and put the board into Recovery Mode

#### Connect Jetson Orin to host

- Connect a USB Type-C cable to Type C Port(10) of Orin and host.

- Plug in power cable into Type C Port (4) of Orin.

#### Put Jetson Orin into Recovery Mode

- Press and hold down the Force Recovery button ( 2 ).

- Press and hold down the Reset button ( 3 ).

- Hold for 1 second.

- Release the Reset button ( 3 ) and then then Force Recovery button ( 2 ).

#### (For WSL2 only) Connect USB devices to WSL2

In Window PowerShell, type the following

#### Confirm if host is successfully connected to Jetson Orin in Recovery Mode

In Ubuntu host, type the following

### Upgrade using SDK Manager

In host terminal, type “sdkmanager” to open the SDK Manager. Login in and follow steps as shown below.

#### Flash OS

3

1

2

2

1

2

1

#### Login into Jetson Orin and Initialize the OS. Connect Jetson to a Wi-Fi / Ethernet.

- Connect Jetson Orin with a monitor, mouse and keyboard. Login in and initialize the Ubuntu system.

- Connect Jetson Orin to the internet.

#### Install Jetson SDK Components through Ethernet using SDK Manager

- Default IP for installed system is 192.168.55.1

1

2

### Check if Jetpack OS is successfully installed

For Jetpakc6.2, L4T36.4.3 is installed as shown below.

### Check if Jetpack SDK Components are successfully installed

For Jetpack6.2, Cuda 12.6 is installed as shown.

## Configurate Additional SSD

Open Disks from Ubuntu Applications and follow instructions below.

### Formula the SSD

### Change Mount Point

### Start the SSD

### Verify SSD Performance

### Access SSD

## Install RT-Kernel

Install RT Kernel using OTA update from here. Be aware of the release version (i.e. r36.4) in the link.

### Verify if RT-Kernel is installed

- Before installation

- After installation